ROBOT FRAME: TORSIONAL STIFFNESS ANALYSIS

- Launch Inspire.

- Click Open Model on the Files icon. In the "Open File" window, select "Robot_Frame. STEP" file and click "Open."

- Go to File > Preferences > Run Options. Then, select "SimSolid" as the analysis solver.

- Deactivate the following three highlighted support parts to exclude them from the analysis.

- Assign the Aluminum material to the Robot Frame.

- Navigate to Structures > Contacts. Ensure that the contacts are "bonded" and not "sliding".

- Go to Structures > Loads > Support. Apply support to any one face to fix it.

- In the Structure tab, under Loads > Forces, apply two equal and opposite forces with a magnitude of 412 N each on the unfixed faces.

- Go to the "Structure" tab, then navigate to the "Analyze" ribbon.

- Ensure that Solution Adaption is set to "For Stiffness (Faster)".

- Click "Run" to initiate the analysis. The "Run Status" window will appear. A green check mark will appear when the analysis is complete (this may take a few minutes).

- Look for the green flag on the "Analyze" tab. Click on it to view the results. The results will be displayed in the Analysis Explorer.

Activate the previously deactivated parts by going to View > Model Configuration. Then, repeat the above steps to perform the torsional analysis.

Analysis Result:

Robot Frame Design 2

Robot Frame Design 1