Student Race Car

The objective of this demo is to provide insight into the typical modeling steps related to the analysis of a student racing car subjected to bending-, shear-, and torsion loads.

Please note, that the load cases, geometry of the frame, the dimensions of the 1D element cross sections, material parameters etc. are just conceptual.

In case you are new to HyperWorks we recommend viewing the video series: HyperWorks Starter Kit first. The StarterKit vides, provide a basic insight into the work flow and philosophy of HyperWorks and is available for free in the Academic Blog:

http://www.altairuniversity.com/2011/09/22/hyperworks-starterkit-video-series/

Also this tutorial is available (in a slightly modified version) as a demo video at:

http://www.altairuniversity.com/2012/03/09/simulation-driven-design-of-a-student-race-car-free-video-series/

Linear Static Analysis – Base Design

The following working steps are briefly shown:

- Model geometry import

- 1D Meshing (CBAR elements)

- 1D Mesh checks

- Material definition (Card Image MAT1)

- Definition of frame properties (Card Image PBARL) & trouble shooting

- Wheel suspension

- Boundary conditions & loads (3 different loadsteps)

- Analysis

- Postprocessing

Right at the beginning of the demo the User Profile is set to OptiStruct

Model Geometry Import

In order to keep this demo simple, the given CAD was updated and improved in order to fix misaligned lines, duplicated lines, and minor asymmetries in the overall geometry (a more faulty geometry would also be available (file: 1D_frame_lines_poorCAD.hm).

Let’s get started with importing the CAD data (file: 1D_frame_lines_CAD.hm) into HyperMesh

The imported model consists of

2 component collectors, named

“lvl1” and “Points”.

Figure: The geometry of the student race car consists of lines only. The crosses displayed at the location of the lines indicate so called „line handles“ which ease viewing and selection of lines.

Figure: The “location” of the wheel suspension is given by pre-defined geometry points (red colored x symbols).

Take care, that the geometry is really symmetrical. The symmetry plane is typically at y=0. Thus, depending on the quality of the given CAD data (here: lines) it may be required to manually trim (split), combine or extend lines (GeometryEditLines….).

In case lines need to be created from scratch, there are many possibilities available

1D Meshing (CBAR elements)

As the given geometry consists of lines the “Line Mesh” functionality is employed for meshing.

At this stage, there is no property collector available (compare with what is listed in the model browser currently). Thus, the field “property” remains blank. The mean “element size” is set to be 100 mm, “the element configuration” is bar2.  Bar2 elements are 1D (1st order) elements with 2 nodes used to model axial, bending, and torsion behavior. We just have to specify the kind of 1D element which should be used for the analysis.

Hence, either before or after meshing check the settings in the “element type” panel:

The element type “bars” may be set to CBEAM (default). As we are going to use CBAR elements it may be necessary to switch the element type in here. Then select all elements and click on update.

In this image the 1D meshed frame is depicted. Elements are displayed in the shrink mode      which temporarily reduces element size by 20%.

Mesh Quality

In a simplified (and certainly not too meaningful way) the element size was chosen to be 100 mmm for the entire frame. What is the minimum element size?

Open the “Element Check” panel by clicking on the symbol

As a “critical” minimum element size (no special reason for this figure) 20 mm is chosen. The check reveals

What is causing these small elements?

To extract the failed elements, click on “save failed” (right after the check was carried out) which places the failed elements in a dedicated user mark:

Then turn off all elements and geometry within the Model Browser (leaving blank screen). In order to retrieve the failed elements from the user mark activate the find option

Then click on „elems“

which opens up the extended selection window. In there select “retrieve”.

This will upload the failed elements into the screen (you may have to hit  to resize the elements)

Figure: In this image the 4 failed 1D elements are depicted. Keep in mind that the size of the failed elements is <3mm, whereas global element size is 100 mm.

Adding line information to the displayed elements indicates that something with the underlying lines is wrong!

Figure: Thick “lines” correspond to the failed 1D elements (displayed in shrink mode), whereas the thin “lines” are true geometry (lines).

As shown in the image above the CAD lines (and hence the CBAR elements) form a squared area (=erroneous) resulting from misaligned lines. In other words, the squared area is a CAD design flaw and needs to be removed. Depending on the project requirement e.g. meshing with different mesh sizes, you may decide to fix (repair) the problem by rebuilding the lines or by editing the elements directly.

In here we will fix the elements directly:

First step: deleting the small elements (length < 2.7 mm) ()

The elements marked with           remain, as indicated in the following figure. Note, the incorrect lines are not modified.

As a result the 1D elements are not coupled anymore (i.e. there is a physical gap). In order to merge both nodes (bold yellow symbols) together, the functionality “replace” is employed (Page 1Dreplace)

In the image below, the thin “lines” represent the initial geometry (=lines), whereas the bold “lines” are the 1D elements after completing the replace operation. The two yellow nodes located at the intersection of the lines indicate the initial position of the nodes. These nodes have been merged into a single node now located right in the middle of the squared area.

The “replace” functionality allows to merge (equivalance) nodes in such a way, that selected nodes move towards each other and meet at their midpoint (half the gap distance). Of course we are introducing an other inaccuracy into the model …

Material Definition

The material to build the frame will be “steel” with the following (default) properties: linear elastic, isotropic and temperature independent. This material behavior is defined as (and through) the Card Image MAT1.

Units are: N, mm2 and t (tons)

Frame Property Definition & Trouble Shooting

For simplicity reasons the frame will be modeled with CBAR elements referencing a “tube” cross-section with constant properties thoughout. The CBAR element properties can be best created with HyperBeam (integrated in HyperMesh). Once the cross section is defined, the cross section information is then stored as a beamsection within a beamsection collector.

The workflow is as:

- Define the required tube cross-section with HyperBeam and store the respective cross section data (area, moment of inertia etc.) in a beamsection collector

- Generate a property collector with Card Image PBARL. Inside this property collector the previously defined beamsection and a material collector is referenced. Eventually, the property collector is then assigned to all CBAR elements

These steps are explained in some detail next:

- Defining the tube cross-section with HyperBeam

HyperBeam can be accessed through the main menuPropertiesHyperBeam

Alternatively, HyperBeam may be started from Page 1 HyperBeam.

Note: As we are using OptiStruct (Radioss bulk) as FEM solver, our life’s becomes easier if we make use of the 1D section library of OptiStruct. Therefore, the “standard section library:” should be OPTISTRUCT, the “standard section type:” should be “Tube”. Other section do exist of course

In the graphic editor of HyperBeam, the chosen tube section is displayed.

For the base design (it is actually just a “guess”) the tubes radii are assumed to be

rout=12.5 mm, rinside=10.5 mm.

Of course, the tubes inner (DIM2) and outer (DIM1) radii can be easily changed and adjusted in HyperBeam.

Based on the cross- section the additional values such as area, moments of inertia etc. are automatically determined and listed in the side bar of HyperBeam. All this information is now readily available as a beamsection stored in a beamsection collector (Note, that different beamsections with different attributes, names and ID’s may reside inside a beamsection collector).

Once the definition of the cross section is completed you can return to the HyperMesh GUI by clicking the icon  beneath “Model”

- Defining a property collector

As before with the material collector, make a right mouse button click in the “Model Browser”

and specify the Card Image PBARL as well as the previously defined material.

The Card image of the property collector (PBARL) includes references to:

- Beamsection (here the ID of the beamsection is 8 (not to be mistaken with beamsection collector) of cross section type (CStype) TUBE. Inner and outer radii are DIM2 and DIM1

- Material (MID=1)

In the next step, this property collector is assigned to all 1D elements. Hereto, the respective functionality available in the Model Browser is used: Right mouse button click on the before defined property collector, then select, Assign.

To better visualize the 1D elements we switch from the “Traditional Element Representation”  to “3D Element Representation” . In addition the element shrink option is activated.

Figure: Car frame meshed with 1D elements (CBARS) with “shrink view”  on. The x-symbols outside the frame refer to geometry points. These points will be used to model the wheel suspension.

Figure: Detailed view of the meshed frame (shrink view mode is active). A uniform tube cross section is used for simplicity reasons (ro=12.5 mm, ri=10.5 mm).

Trouble Shooting

However, it may happen that some 1D elements are not properly displayed. What is the problem?

Figure: The depicted “line” is actually a CBAR element.

Likely causes & ways to solve this issue

- Make sure that a “property” is assigned to ALL the 1D elements. This can be checked visually in many ways, for instance, by activating the element display mode:

If a property collector is assigned to the 1D elements they will be displayed with the same color as the property collector.

- Check the properties of the 1D elements again i.e. review the property collector (in the Model Browser make a right mouse button click on the corresponding property collector --> Card Edit)

- Check the element orientation.

The element orientation is defined through its local xy-plane. The local element xy-plane in turn is defined through its x-axis (longitudinal direction from node a to b) and a vector v. Basically, one needs to take care of the v vector in order to solve (determine) the element orientation!

(more information is also available in the help documentation \help\hwsolvers\hwsolvers.htm, then Index > CBEAM or CBAR)

To control (or update) the local element xy-plane and thus the v vector go to the bars panel (main menuMeshEdit1D ElementsBarsUpdate)

Select any of the displayed 1D elements (in the image below the “3D Element Representation”  is active)

As shown above this will display not only the local element coordinate system but also the components of the vector v (with respect to the global coordinate system). Here, the vector v parallels the global y-direction(y comp = 1). The element xy-plane thus parallels the global xy-plane.

In the next step, we are reviewing the element orientation of a 1D element which is not properly displayed (cross-sections are not shown).

Just repeat the “review steps” from before …

This time the location of node a and b is shown only. In the panel shown below it becomes apparent that the components of the v vector are all zero.

The good thing: the components of vector v can be corrected/updated at any time.

Select a single (or multiple elements), then specify the component of the vector v with respect to the global (basic) coordinate system. In case the y-component of vector v is set to y=1 (global y), both, vector v and local y-direction of the element coincide. In the image below the local y direction and v vector are superimposed on each other.

In the image below, the vector v doesn’t coincide with any of the global components. Now it is quite apparent, that vector v and local x-axis together specify the local xy-plane of the element.

The same element is viewed from a different direction

Wheel Suspension

Rigid links (RBE2) will be used to model a simplified wheel suspension. In the image below the modeled wheel suspension is depicted. Note the location of the independent (I) and dependent (D) nodes, respectively.

In the first step temporary nodes are created at the location of the geometry points (stored in the component “points” and marked as x in the figure below) which will serve as independent nodes.

Before building rigid links corresponding component collectors (no Card Image needed) should be created (i.e. rigid_up and rigid_low)

Building rigid elements:

Note that the panel next to “dependent” is set to “nodes” as multiple dependent nodes will be referenced.

Note the location of the independent and dependent nodes.

BC’s and Loads

Three different loadsteps are considered:

- Torsion

- Bending

- Shear

In all three loadsteps the frame is constraint in the same way (see image below). Thus, 4 loadcollectors (no Card Images needed) must be created.

Constraints

The corresponding nodes are fixed with respect to translational and rotational displacements (dof 123456).

Torsion

Torsion is imposed through vertically applied forces. Of course the referenced forces and magnitudes are just “dummy” assumptions. The frame is constraint as described before.

The corresponding torsional loadstep is defined in the ”LoadStep” panel.

Bending & Shear

The working steps related to the setup of the bending and shear load cases are pretty much the same. The orientation of the bending and shear forces is shown in the images below. Care must be taken, that these forces are stored in their respective load collectors (no Card Image needed). The loadstep named “Bending” then references the loadcollectors “Bending” and “Constraint”; the loadstep “Shear” the loadcollectors “Shear” and “Constraint”.

Load case Bending

Load case Shear

This completes the model set-up. The model is saved as a binary *.hm file (including CAD information) and then exported as an OptiStruct analysis deck (ASCII format).

Analysis

As the UserProfile (hence the FEM solver) is chosen to be OptiStruct, we start the analysis from within the panel OptiStruct

Take care and don’t forget to add the file extension *.fem. The “run options” is set to “analysis”.

Before visualizing displacements and stresses, check the *.out file located in the directory where the analysis file was saved in.

In the outfile a model summary, estimated memory, likely warning and errors etc. are documented. Hint: The most typical error messages and debugging steps related to a linear static analysis are discussed in the StarterKit video series.

Postprocessing

Finally, we are there – the FEM solver OptiStruct reports: solution is completed.

The results shown below are just “conceptual”. The detailed evaluation of the car frames performance (i.e. displacements and stresses) is deliberately left to the “reader” of this tutorial.

Bending loadstep

Bending loadstep: Displacements are scaled by factor 50. Undeformed structure is displayed in gray.

In order to display element stresses one needs to recall the definition and location of the stress recovery points of the CBAR element.

Of course, the orientation of the CBAR element now determines whether for instance recovery point C experiences tension or bending. Furthermore, it is important to realize that the stress value at the node of consideration (for instance node A), is then used to contour the entire element!

Shear loadstep

Shear loadstep: Displacements (in mm) are scaled by a factor 50. Undeformed structure displayed in gray

Torsion loadstep

Torsion loadstep: Displacements (in mm) are scaled by a factor 50. Undeformed structure displayed in gray

Summary Base Design:

Total mass: 42.1 kg

ro= 12.5 mm

ri=10.5 mm (Note, the initial radii used in the base design are just a first guess).

| Load Step | Max. Displacement (mm) | Node |
| --- | --- | --- |
| Bending | 1.45 | 2764 |
| Shear | 2.03 | 2678 |
| Torsion | 6.29 | 2085 |

Appendix - Quiz: Torsion loadstep. Do you see any errors in here?

Torsion load case: Displacements scaled by a factor 10.

One lesson to be learned: Not every modeling mistake necessarily causes the FEM program to abort its run.